Republic of Iraq

Ministry of Higher Education

and Scientific Research

Al-Mustaqbal University College

Computer Engineering Techniques Department

(عملي)

Subject:  Digital Signal Processing

Third stage

Experiment No. 4

By

Sarah Abbas & Raya A. hadawi

Experiment No.4

Name of Experiment:  LINEAR  CONVOLUTION

Aim:  Write a MATLAB program to compute linear convolution of two given sequences.

Matlab code

clc;
clear all;
close all;

x1 = input ('enter the first sequence ');

subplot (2,2,1);

stem (x1,'r');

ylabel ('amplitude');

title ('plot of the first sequence');

grid on;

x2 = input ('enter 2nd sequence ');

subplot (2,2,2);

stem (x2, 'r');

ylabel ('amplitude');

title ('plot of 2nd sequence');

grid on;

f = conv (x1,x2);

disp ('output of linear conv is');

disp (f);

subplot (2,2,3);

stem (f,'r');

xlabel ('time index n');

ylabel ('amplitude f');

title('linear conv of sequence');

grid on;

Result

Output:

enter the first sequence [1 2 3 4]

enter 2nd sequence [2 -1 2 3]

output of linear conv is

2 3 6 12 8 17 12