Lecture 3.  /B

# Calculation of X-ray wavelengths & frequency

If electrons are accelerated to a velocity v by a potential difference V and then allowed to collide with a metal target, the maximum frequency of the X-rays emitted is given by the equation:

½ mv2= eV = hf

Therefore:

X ray frequency (f) = eV/h

↨↨
This mathematical  formula  shows that the maximum frequency is directly proportional to the accelerating voltage.

Example problem
Q/Calculate the minimum wavelength of X-rays emitted when electrons accelerated through 30 kV strike a target.

Answer is:-
f = [1.6 x 10-19 x 3 x 104]/ 6.63 x 10-34 = 7.2x1018 Hz

Therefore the wavelength

 (= c/f) is 0.41 x 10-10 m = 0.04 1 nm (compared with some 600 nm for yellow light)

(Quizzes examination No. 1) Q1/ Calculate the: - 1-maximum energy   &     2- minimum wavelength

for an x-ray beam generated at 100 kVp.

Answer /

The maximum energy (keV) numerically equals the maximum tube voltage (kVp).
Because the maximum tube voltage is 100 kVp, the maximum energy of the photons is 100 keV:

λ min= {1.24}/{100 kVp}

λmin=0.0124 nm

# Q2/

# Find the

# (a) maximum frequency

# (b) Minimum wavelength of X-rays produced by 30 kV electrons.

The Solutions

## (a).

## Potential of the electrons, V=30kV=3×104V

## Hence, energy of the electrons, E=3×104eV

## Where,

## e = Charge on an electron = 1.6×10−19C

## Maximum frequency produced by the X-rays is ν

## The energy of the electrons is given by the relation is E=hν

## Where,

## h = Planck’s constant = 6.626×10−34Js

## ν=E/h=7.24×1018Hz

## (b).

## Energy of a electron, E=30×103eV

## Let Maximum frequency produced by the X-rays be ν

## ν=E/h=1.6×10−19×3×104/6.626×10−34=7.24×1018Hz where h is the planck's constant.

## The minimum wavelength produced by the X-rays is given as-

## λ=c/ν=0.0414nm