Analysis and Design of

# Reinforced Concrete Structures

Presentation Overview

- Building system primary functions

- Types of load

- RC structural systems

- RC structural members

2

## Basic Building System Functions

Support gravity loads for strength and serviceability during:

- Normal use (service) conditions

- Maximum considered use conditions

- Environmental loading of varying intensities

3

Vertical deflection (sag)	Lateral deflection (sway)

Dead, Live, etc.

Wind or earthquakes

Performanc	Based Design	Control displacements within acceptable limits during service loading, factored loaded, and varying intensities of environmental loading

4

## Types of Load

Gravity:	Lateral

Dead Live

Wind Earthquake

Impact	Soil lateral pressure

Snow Rain/floods

Thermal Centrifugal

# RC Structural Systems

  - Floor Systems

  - Lateral Load Systems

# Floor Systems

- Flat plate

- Flat slab (w/ drop panels and/or capitals)

- One-way joist system

- Two-way waffle system

Flat Plate Floor System

Slab-column frame system in two-way bending

Plan	Elevation

8

# Flat Plate Floor System

Advantages:

- Simple construction

- Flat ceilings (reduced finishing costs)

- Low story heights due to shallow floors

Typical Applications:

- Short-to-medium spans with light loading

- For LL=50 psi, 15’ - 30’ spans

- For LL=100 psi, 15’ – 25’ spans

9

Plan	Elevation

10

Advantages:

- Same as flat plate system, plus

  - Increased gravity and lateral load resistance

  - Increased torsional resistance

  - Decreased slab edge displacements

Typical Applications:

- Same as flat plate systems

11

# Flat Plate w/Beams Floor System

Two-way bending

Gravity and lateral load frames

12

# Flat Plate w/Beams Floor System

Advantages:

- Increased gravity and lateral load resistance

- Simple construction

- Flat ceilings (reduced finishing costs)

Typical Applications:

- Medium spans with light loading

- For LL=50 psi, 25’ - 30’ spans

- For LL=100 psi, 20’ – 30’ spans

13

# Flat Slab Floor System

Flat plate with drop panels,shear capitals, and/or column capitals

Plan	Elevation

14

# Flat Slab Floor System

Advantages:

- Reduced slab displacements

- Increased slab shear resistance

- Relatively flat ceilings (reduced finishing costs)

- Low story heights due to shallow floors

Typical Applications:

- Medium spans with moderate to heavy loading

- For LL=50 psi, 30’ – 35’ spans

- For LL=100 psi, 25’ – 35’ spans

15

# One-Way Joist Floor System

Rib (joist) slab : (One-way bending)

2D gravity or lateral frames

2D lateral frames

Floor joists, type

16

# One-Way Joist Floor System

Rib (joist) slab with beams: (One-way bending)

Lateral space frame

Floor joists, type

17

# One-Way Joist Floor System

Typical Joist

Top of Slab

Width varies 4”, 6” or larger

8-24” for 30” Modules

16-24” for 53” Modules

1:12 Slope, type	14-24” for 66” Modules .

- 2’ or 3’ cc. – Joists

  - 4’ or 6’ cc. – Skip joists

  - 5’ or 6’ cc – Wide-module joists

# One-Way Joist Floor System

Advantages:

- Longer spans with heavy loads

- Reduced dead load due to voids

- Electrical,

mechanical etc.

can be placed between voids

- Good vibration resistance

Typical Applications:

- Medium-to-long spans with heavy loading

- For 30” modules, 35’ – 40’ spans

- For 53” & 66” modules, 35’ – 50’ spans

# Two-Way Joist Floor System

Waffle slab : (Two-way bending)

2D lateral frames

Waffle pans, type

20

# Two-Way Joist Floor System

Advantages:

- Longer spans with heavy loads

- Reduced dead load due to voids

- Electrical,

mechanical etc.

can be placed in voids

- Good vibration resistance

- Attractive Ceiling

Typical Applications:

- Long spans with heavy loading

- For 3’, 4’, and 5’ modules, 40’ – 50’ spans and beyond

# Floor System Effective Cost

(PCA 2000)

Live Load, psf

100

Flat Plate

Flat Slab

One-way joist

50

25	30	35	50