الفصل الاول / المحاضرة الاولى

تقنية صناعة الخرسانة 025/024

Definition of Concrete

Concrete is a mixture of cement (11%), fine aggregates (26%),coarse aggregates (41%) and water  (16%)   and air (6%)

Cement   = powder

Cement + water = Cement Paste

Cement Past + Fine Aggregate ( FA) = Mortar

Mortar Paste +Coarse Aggregate  = Concrete

Produced concrete

Portland cement,  water,  sand, and  coarse aggregate are proportioned and mixed to produce concrete.

Definition of Cement

Portland cements are hydraulic cements, meaning they react and harden chemically with the addition of water. Cement contains limestone, clay , cement rock ,  and iron , are blended  (  تخلط او تمزج )   and heated to 1200 to 1500 C°.

The resulting product  "clinker".  Gypsum is added to control setting time .

Definition of Fine Aggregate

Normally called sand, this component  can be natural sand or  crushed stone,

The fin aggregate represents particles smaller than 4.75 mm.

Definition of Coarse Aggregate

May be either gravel  or  crushed stone. comprised of particles equal or greater  than  4.75 mm.

Chemical Admixtures

(Materials added to alter تغيير  the properties )

of concrete including  :

• Air entrainment

• Set accelerators    (انجماد الاسمنت)  مسرع تماسك الاسمنت

• Set retarders  : مواد معوقة للانجماد

• Water reducers  خليط مضاف مخفض للماء

*Air entraining admixtures add microscopic air

bubbles to the concrete, enhancing its

resistance to freeze/thaw cycles and makes

the concrete easier to finish.

ظاهرة التجمد والذوبان في الخرسانة :

من الظواهر الطبيعية الخطيرة والتي تؤثر على خصائص الخرسانة المتصلبة والتربة المستخدمة في أعمال الطرق و السدود و الحوائط  الإسنادية وغيرها . وتتخلص ظاهرة التجمد والذوبان في أن وجود المياه في الفراغات البينية سوف يؤدي إلى تجمدها، و زيادة حجمها في فصل الشتاء و فترات الصقيع عندما تنخفض الحرارة تحت الصفر و بالتالي زيادة حجم الفراغات ، و عند الذوبان في فصل الصيف سوف تبقى الفراغات على حجمها ، و في الشتاء التالي سوف تمتلئ هذه الفراغات مرة أخرى بالماء ، وسوف يتجمد في فترات الصقيع مما يؤدي إلى زيادة اكبر في حجم الفراغات. وتستمر هذه العملية بشكل دوري مع فترات الصيف والشتاء ، سوف تؤدي إلى زيادة الاجهادات على التربة والخرسانة، والتي قد تؤدي في لحظة من اللحظات إلى انهيار إنشائي في الخرسانة أو في التربة

*   Set accelerators speed the set-time of the mixture,  enabling finishing operations to begin sooner   during cold weather . تمكين عمليات التشطيب أن تبدأ في وقت اقرب خلال عمليات التشطيب

* Set retarders :   Have the opposite effect, slowing the set and enabling delivery to distant sites and finishing during hot weather.

* Water reducers are used to reduce the amount of water required to produce a given slump. يتم استخدام مخفضات المياه لتقليل كمية المياه المطلوبة لإنتاج هبوط معين They also making the concrete easier to finish, and produce better cement hydration ترطيب الاسمنت او التفاعل بين الاسمنت و الماء و بقية المركبات

. By reducing the amount of water required, cement amounts can be reduced because concrete strength is directly related to the water/cement ratio.

* Definition of Mineral Admixtures

Mineral admixtures include fly ash,  رماد متطايرhydrated lime, silica fume

غبارالسيليكا   and ground blast   furnaceفرن slag خبث .

Many of these materials have like  properties of cement, increase زياده the

strength and density of the concrete. The generally improve the workability, density and long-term strength of concrete.

Definition of Synthetic Fibers  الالياف الصناعية

These are thin polypropylene fibers used as secondary reinforcement. They help

control shrinkage cracking and provide  some impact resistance.

General properties of concrete

Advantages of Concrete

* Concrete has many environmental advantages, including durability ديمومه, heat storage capability, and chemical inertness  الخمول الكيميائي   .

* Ability to be Cast to forming any shape.

* Fire resistant مقاومة الحريق

*On – site  fabrication  يمكن صناعته داخل الموقع

* The raw materials used in cement production are

widely available in great quantities.

*Needs little or no finish or final treatments.

* Low maintenance.

* Can be reused or recycled.

* Concrete can be reused with bituminous asphalt as

road base materials.

Limitations of Concrete

* Low tensile strength

* Low ductility

* instability Volume      عدم استقرار الحجم

QUESTIONS :

Choose the correct answer :

A :  Aggregate occupies a volume of concrete at least equivalent to : 75% , 30%  ,10% , 45%

B:  Which one has a higher volume  stability :  Aggregate , sand , cement  , clay

C:  Aggregate used with Cement Paste because :  Inert, expensive materials ,

decrease the stability of cement paste , not Inert  material