Example of calculating the coordinates of a closed polygon :

The lengths and angles of a closed polygon were measured and the results were as follows: If side AB is located in the first quadrant, halfway between north and east (Az. Of AB = 45° 0' 0'') and the coordinates of point A are equal to (YA = 1000 1000 XA =). Calculate the corrected coordinates of the polygon using Baudage's correction method.

C 84° 57' 12'' 106° 40' 21'' 170.88 m 146.65 m B A 244.61 m 66° 53' 21'' 217.09 m D 101° 29' 14''

المجموع النظري للزوايا = ( 360° 0' 0'' )

مجموع الزوايا المقاسة حقلياً = 106°40'21'' + 101°29'14'' + 66°53'21'' + 84°57'12''

=  360° 0' 8''

الخطأ = المجموع الهندسي للزوايا – مجموع الزوايا المقاسة حقلياً

الخطأ = ( 360°0'0'' ) – ( 360°0'8'' )

الخطأ =  - 8''

التصحيح = ( الخطأ / عدد الزوايا ) = ( -8 / 4 ) =  -2'' لكل زاوية

الان نحسب الزوايا المصححة :

الزاوية المصححة = الزاوية المقاسة حقلياً ± مقدار التصحيح

Corrected Angles :

A= 106° 40' 19''                                          C = 66° 53' 19''

B= 101° 29' 12''                                          D = 84° 57' 10''

AZ. of AB = 45° 0' 0''

AZ. of BC = AZ. of AB + Corr. Angle at B ±180°

= (45° 0' 0'' ) + (101° 29' 12'' ) ± 180°

= (146° 29' 12'' ) ± 180° = 326° 29' 12''

AZ. of BC = 326° 29' 12''

AZ. of CD = AZ. of BC + Corr. Angle at C ± 180°

= 326° 29' 12'' + 66° 53' 19'' ± 180°

= 393° 22' 31'' ± 180°

= 213° 22' 31''

AZ. of DA = AZ. of CD + Corr. Angle at D ± 180°

= 213° 22' 31'' + 84° 57' 10'' ± 180°

= 298° 19' 41'' ± 180°

= 118° 19' 41''

AZ. of AB = AZ. of DA + Corr. Angle at A ± 180°

= 118° 19' 41'' + 106° 40' 19'' ± 180°

= (225° 0' 0'' ) ± 180°

= 45° 0' 0''

الان نحسب المركبات :

For line AB :

∆x AB = LAB * SinAZ of AB = 146.65 * Sin (45°0'0'' ) = 146.65 * 0.707 = 103.7m

∆y AB = LAB * CosAZ of AB = 146.65 * Cos(45°0'0'' ) = 146.65 * 0.707 = 103.7m

For line BC :

∆x BC = LBC * SinAZ of BC = 217.09 * Sin (326°29'12'' ) = -119.86 m

∆y BC = LBC * CosAZ of BC = 217.09 * Cos(326°29'12'') = 181 m

For line CD :

∆x CD = LCD * SinAZ of CD = 244.61 * Sin (213°22'31'' ) = -134.564 m

∆y CD = LCD * CosAZ of CD = 244.61 * Cos(213°22'31'') = -204.27 m

For line DA :

∆x DA = LDA * SinAZ of DA = 170.88 * Sin (118°19'41'' ) = 150.42 m

∆y DA = LDA * CosAZ of DA = 170.88 * Cos(118°19'41'') = -81.09m

الان نحسب مقدار الخطأ الافقي والعمودي

Ex = ∆x AB + ∆x BC + ∆x CD + ∆x DA

Ex = (103.7) + (-119.86) + ( -134.564) + (150.42)

Ex = -0.3m

Ey = ∆y AB + ∆y BC + ∆y CD + ∆y DA

Ey = -0.66 m

ايجاد مقدار التصحيح لكل مركبة افقية وعمودية :

تصحيح الضلع AB :

∆x AB ) correct to AB =  LAB L*Ex)

∆x AB ) correct to AB =  146.65779.13*0.3)

(∆xAB) correct to AB  = 0.056m

∆y AB ) correct to AB =  LAB L*Ey)

∆y AB ) correct to AB =  146.65779.13*0.66=0.124 m)

تصحيح الضلع  BC :

∆x BC ) correct to BC =  LBC L*Ex)

∆x BC ) correct to BC = 217.09779.13*0.3=0.084)

∆y BC ) correct to BC =  LBC L*Ey)

∆y BC ) correct to CD = 217.09779.13*0.66=0.184m)

تصحيح الضلع  CD :

(∆xCD) correct to CD  = 0.094m

(∆yCD) correct to CD  = 0.207m

تصحيح الضلع  DA :

(∆xDA) correct to DA  = 0.066 m

(∆yDA) correct to DA  = 0.145 m

الان نحسب القيم المصححة للمركبات

الضلع AB

(∆xAB) corrected  = 103.7 + 0.056 = 103.756 m

(∆yAB) corrected  = 103.7 + 0.124 = 103.824 m

الضلع BC

(∆xBC) corrected  = (-119.86)  + 0.084 = -119.776 m

(∆yBC) corrected  = 181 + 0.184 = 181.184 m

الضلع CD

(∆xCD) corrected  = ( -134.56 )  + 0.094 = -134.466 m

(∆yCD) corrected  = (-204.27)  + 0.207 = -204.063 m

الضلع DA

(∆xDA) corrected  = (150.42 )  + 0.066 = 150.50 m

(∆yDA) corrected  = (-81.09 )  + 0.145= -80.945 m

بعد انت انتهينا من حساب تصحيح المركبات سوف ننتقل الى حساب الاحداثيات

For Line AB

XB = XA + (∆xAB) corrected

XB = 1000 + 103.756 = 1103.756 m

YB = YA + (∆YAB) corrected

YB = 1000 + 103.824 = 1103.824 m

For Line BC

XC = XB + (∆xBC) corrected

XC = 1103.756 + ( -119.776 ) = 983.98 m

YC = YB + (∆YBC) corrected

YC = 1103.824 + 181.184 = 1285.008 m

For Line CD

XD = XC + (∆xCD) corrected = 983.98 + ( - 134.466 ) = 849.51m

YD = YC + (∆YCD) corrected = 1285.008 + ( - 204.063 ) = 1080.945m

For Line DA

XA = XD + (∆xDA) corrected = 849.51 + 150.49 = 1000 m

YA = YD + (∆YDA) corrected = 1080.945 + ( - 80.945) = 1000 m

| Line | Length | Corrected Angle | Az. | ∆x = L * Sin (AZ.) | ∆y = L * Cos(AZ.) | Cor. to ∆x | Cor. to ∆y | Corrected ∆x | Corrected ∆y | Coordinates ( X , Y ) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| AB | 146.55 | 101° 29' 12'' | 45° 0' 0'' | 103.7 | 103.7 | 0.056 | 0.124 | 103.75 | 103.824 | A ( 1000 , 1000 ) |
| BC | 217.09 | 66° 53' 19'' | 326° 29' 12'' | -119.86 | 181 | 0.084 | 0.184 | -119.776 | 181.184 | B ( 1103.75, 1103.824) |
| CD | 244.61 | 84° 57' 10'' | 213° 22' 31'' | -134.56 | -204.27 | 0.094 | 0.207 | -134.466 | -204.063 | C (983.98 , 1285.008) |
| DA | 170.88 | 106° 40' 19'' | 118° 19' 41'' | 150.42 | -81.09 | 0.066 | 0.145 | 150.49 | -80.945 | D (849.51, 1080.945) |
| المجموع | 779.13 |  |  | - 0.3 m | - 0.66 | +0.3 | +066 |  |  | A (1000 , 1000 ) |