A

Q1/Assume that SP = FF20H, AX = FH, BX = 3CH, and CX = 9BAH DX=ABCD. Find the content of the stack and stack pointer after the execution of each of the following instructions. PUSH AX, PUSH BX, PUSH CX.

Q2/ Find the status of the condition flags for the following operations, why? MOV DH, C9H MOV DL, 30H ADD DX, 1BD0H.

Q3/Suppose that DS = 120EH, SS = 1000H, EAX = 00001000H, EBX = 0000200FH, ECX = 00003000H, BP = 0100H, and SI = 0250H. Determine the physical address for each of the following instructions, assuming real mode operation:

(a) MOV DL, [800H]           (b) MOV [SI+500H], EDX            (c) MOV [BP+SI+10], AL
Q4/ Choose the correct answer from the following:

1. The two units in the 8086up are: 
         a- CU& PU.         b- EU&CU.         c- BIU&EU.        d- BIU&PU.

2. If the CS =FF59, the range of physical address is
        a- FF590....0F58F.     b- 0F58F.....0F5FF.       c- FF950.....FF9FF.        d- 0F590....0F59F.