Shear force, Bending Moment and Axial Force Diagrams.

For the strip of width (dx) infinitesimal element

fy=0

V + dv = v + w *dx

W= dvdx      ------------- 1

Slope of shearing force diagram of any section equal to the intensiting the load at that section.

By integration:-

v1v2dv= x1x2 wx.dx

Vv1v2 = x1x2 wx.dx

V2- V1 = x1x2 wx.dx

∆V=x1x2 wx.dx  --------------2

Changing in shearing force between x1&x2 section equals the total applied load between x1 &x2.

MO=0

M + dM = M + Vdx + Wx dx22

Wx dx22, neglect because is small magnitude

dM = v . dx

Vx = dM dX ---------------- 3

Slope of bending moment diagram at any section equals the shear force at that section.

By integration:-

M1M2dM= x1x2 Vx.dx

MM1M2 = x1x2 Vx.dx

M2- M1 = x1x2 Vx.dx

∆M=x1x2 Vx.dx  -------------- 4

Change in bending moment diagram between section x1 & x2 equal to the area under (v – diagram) between x1 & x2 plus any additional concentrated moment applied between x1 & x2.

Differentiate Equation:-

With respect to x

dvdx=d2Mdx2, Then sub into equ. 1

W = d2Mdx2 ------------ 5

Sign Convention:-

- Internal force:-

- Axial:   tensile   +ve

Comp.    –ve

- Shear:   on left segment

Down word +

Up word -

- Bending Moment:

- External force:-

- Up word force +ve

- Clockwise moment +ve

- Axial force to the left +ve

- Curves:-

EX1:- Draw the axial force; shear force and bending moment diagrams.

A1= 1/ (n+1), الكيرف مقعر     A2= n / (n+1), الكيرف محدب

EX2:  Draw the axial force; shear force and bending moment diagrams for the frame shown below.

Sol: - SEGMENT (AB)

Segment (DC)

Segment BCE

EX3:- Draw the axial force; shear force and bending moment diagrams for the frame as shown below.

Sol:

MA=0

Cy*6 – (120*2) =0, Cy= 40 kN

Fy=0, 40 + Ay = 0, Ay= -40 kN = 40 kN

FX=0, 120 – Ax = 0, Ax = 120 kN

For segment. AB

Fy=0, -40 + By = 0, By = 40 kN

FX=0, 120 – 120 – Bx = 0, Bx = 0

MB=0, MB + 120*2 + 40*3 -120*4 =0

MB= 120 kN.m

Segment AB.

Segment BC

EX4:  Draw the axial force; shear force and bending moment diagrams for the frame shown below.

Sol:

For segment (DEF)

MD=0,

Fy * 2 – (50*1) =0

Fy= 25 kN

For whole structure

MA=0 => Fx*6 + (25*4) – (50*3) – (10 *1) – (40*3) =0

Fx = 30 kN

Fy=0,

Ay +25 -50- (5*2) = 0, => Ay= 35 kN

Fx=0, => -Ax – 30 + 40 = 0, => Ax= 10 kN

Segment ABC

Segment CDEF

EX5:  Draw the axial force; shear force and bending moment diagrams for the frame shown below.

Sol:-

For Member AB:

For Member CD:

For Member BC:

H.W:  Draw the axial force; shear force and bending moment diagrams for the   frame shown below.