Lecture  5

It is a device used to sterilize equipment’s by subjecting them to high pressure saturated steam at 121 °C for around 15–20 minutes, this means that all bacteria, viruses , fungi and spores are inactivated.

The sterilizing action depend on the heat of pressurized steam which is about 2.2 bar at 121°C , ( when heat water in a closed container like autoclave we can raise the temperature of water above boiling temperature).

- Strong metal jackets ( strong enough to withstand high pressure required).

- Autoclave door.

- Pressure gauge (indicate pressure).

- Thermostat.

- Safety valve.

- Plastic rubber.

- Timer.

- Water level indicator.

- Thermometer (give temperature).

Electrical	heater	(heat	supply)	located	at	the	bottom of autoclave which is covered with water.

**Autoclaves	are	widely	used	inmicrobiology,medicine, Typical loads include laboratory glassware, surgical instruments and medical waste.

- Fill the bottom of autoclave with distilled water.

- Place the object within the basket.

- close the lid and tighten the screws.

- adjust the temperature, pressure and time.

- Upon completion of the sterilization time, discards steam by opening the valve then opens the lid.

Shape (1) : Autoclave devic