University of Al-Mustaqbal

College of Science

Department of Medical Physics

Electricity

Lect. Dr Duaa jaafer Al-fayadh

2024-2025

first stage

Capacitance

Lecture Eight

Outline

- Capacitance

- Calculating the Capacitance

- Energy Stored in an Electric Field

- References

- Capacitance

The capacitor, a device in which electrical energy can be stored. For example, the batteries in a camera store energy in the photoflash unit by charging a capacitor.

Figure 1 shows some of the many sizes and shapes of capacitors. Figure 2 shows the basic elements of any capacitor—two isolated conductors of any shape. No matter what their geometry, flat or not, we call these conductors plates.

Figure 25-3a shows a parallel-plate capacitor, consisting of two parallel

conducting plates of area A separated by a distance d. The symbol we use to

represent a capacitor ( 	).

Figure 1: An assortment of capacitors. Two conductors, isolated electrically from each other. (a) A parallel-plate capacitor, (b) As the field lines show

When a capacitor is charged, its plates have charges of equal magnitudes but opposite signs: +q and -q.

The	charge	q	and	the	potential	difference	V for	a	capacitor	are proportional to each other; that is:

The proportionality constant C is called the capacitance of the capacitor. The SI unit of capacitance is the coulomb per volt.

This unit occurs so often that it is given a special name, the farad (F): 1 farad = 1 F = 1 coulomb per volt = 1 C/V.

- Calculating the Capacitance

To calculating the capacitance (C): (1) Assume a charge q on the plates;

(2) calculate the electric field between the plates, using Gauss’ law; (3) calculate the potential difference V between the plates.

Letting V represent the difference Vf - Vi:

Example: Find the capacitance for the parallel plates of area A = 0.5 m2 separated by a distance d = 10 cm?

Solution:

- Energy Stored in an Electric Field

The energy is stored in the electrical field in the space between the capacitor plates. It depends on the amount of electrical charge on the plates and on the potential difference between the plates. The work required to bring the total capacitor charge up to a final value q, this work

is stored as potential energy U in the capacitor:

The potential energy of a charged capacitor may be viewed as being stored in the electric field between its plates.

Example: An capacitor plates has a capacitance C = 1.25 F, how much potential energy is stored in the capacitor plates when potential difference between the plates V= 5mv?

Solution:

- Refrences

Walker, Jearl, Robert Resnick, and David Halliday. Halliday and resnick fundamentals of physics. Wiley, 2014.