1

[ Toturial  Sheet ]

Q1. The maximum peak-to-peak voltage of an AM wave is 16 mV and the minimum peak-to-peak voltage is 4 mV. Calculate the modulation factor.

Q2. A carrier of 100V and 1200 kHz is modulated by a 50 V, 1000 Hz sine wave signal. Find the modulation factor.

Q3. An AM wave is represented by the expression :
[ v = 5 (1 + 0.6 cos 6280 t) sin 211 × 104 t volts ]
(i) What are the minimum and maximum amplitudes of the AM wave?
(ii) What frequency components are contained in the modulated wave and what is the amplitude of each component?

Q4. A sinusoidal carrier voltage of frequency 1 MHz and amplitude 100 volts is amplitude modulated by the sinusoidal voltage of frequency 5 kHz producing 50% modulation. Calculate the frequency and amplitude of lower and upper sideband terms.

Q5.A carrier wave of frequency 10 MHz and peak value 10V is amplitude modulated by a 5- kHz sine wave of amplitude 6V. Determine (i) modulation factor (ii) sideband frequencies and (iii) amplitude of sideband components. Draw the frequency spectrum.

Q6. A carrier wave of 500 watts is subjected to 100% amplitude modulation. Determine : (i) power in sidebands (ii) power of the modulated wave.

Q7. A 50 kW carrier is to be modulated to a level of (i) 80% (ii) 10%. What is the total sideband power in each case ?

Q8. A 40kW carrier is to be modulated to a level of 100%.
(i) What is the carrier power after modulation?
(ii) How much audio power is required if the efficiency of the modulated RF amplifier is 72%?

Solution : (i) Since the carrier itself is unaffected by the modulating signal, there is no change in the carrier power level.

Q9. An audio signal of 1 kHz is used to modulate a carrier of 500 kHz. Determine (i) sideband frequencies (ii) bandwidth required.

Q10. The load current in the transmitting antenna of an unmodulated AM transmitter is 8A. What will be the antenna current when modulation is 40%?