Analog Communication

Second Stage

2024-2025

Experiment No. : 1

Experiment Name:

Amplitude Modulation And De-modulation

M.Sc. Aya Althahab                                                                                                                          Eng. Zainab Fhayer

Amplitude Modulation and Demodulation

## Object :

To Study an amplitude modulated signal, Comparing the modulated signal and the demodulated wave envelope.

## -Tools :-

Computer by using Maltisim program

- function generator , resistor , capacitor , channel oscilloscope , divider , multiplier  summer , DC-power

THEORY:

Amplitude modulated signal can be represented by the following equation:

V (t)= Vc (1+ Vm (t)) COS Wc t

Where Vm (t) is the message signal that is modulating the carrier Vc COS Wct . For simplicity, let us assume that the message signal is a monotone given by the following equation:

Vm (t)= Vm COS Wmt

The amplitude modulated signal can now be written

as: V (t)=Vc(1+Vm  COS Wm t) COS Wct

=Vc COS Wc t +Vc  Vm /2 {COS(Wc-Wm)t+ COS(Wc+Wm)t}

The following figure shows the modulated signal in both time and frequency domains.

Amplitude Modulation is the process of changing the amplitude of the radio frequency (RF) carrier wave by the amplitude variations of modulating signal”

The carrier amplitude varied linearly by the modulating signal which usually

consists of a range of a audio frequencies. The frequency of the carrier is not

affected

Demodulation is the reverse process (to modulation) to recover   the message signal m(t) or d(t) at the receiver.

- Fundamentals of Analog Communications

Analog Communication is an information transmitting mechanism, i.e. music, voice, and video using broadcast radio, or cellular radio, and broadcast television. The significant invention made by Marconi in 1895 was a radio. Later, the foundation of Trans-Atlantic Communication Systems had been taken place. Although digital communications systems are much more efficient, cost-saving, more reliable, some communication systems are still analog.

Figure 1: Basics of Analog Communications

Analog communication techniques can be summarized as:

figure 2: Analog Modulation Techniques

### Demodulation

For AM demodulation, we will examine the Square-Law and Envelope Detector techniques.

### Demodulation by Squaring

Practical Part :

By using MULTISIM connect the circuit of Amplitude Modulation  show in a figure (1&2)

(1)

(2)

Amplitude Demodulation