Retention is the ability of a removable partial denture to resist dislodging forces during function.

Bracing: it is the resistance to horizontal components of masticatory force

Types of retention in a removable partial denture

Direct retainer: is a unit of a removable partial denture that engages an abutment tooth in such a manner to resist displacement of the prosthesis away from basal seat tissues.

Extracoronal retainers

Suprabulge clasp: a removable partial denture retentive

clasp that approaches the retentive undercut from an occlusal or suprabulge direction.

Infrabulge clasp: a removable partial denture retentive

clasp that approaches the retentive undercut from a cervical or infrabulge direction.

Retention is provided by the retentive arm which prevents the partial denture from displacement away from the tissues toward the occlusal.

While the partial denture is seated the retentive tip must be passive. However, it should touch the tooth in the undercut area.

Factors that determine the amount of retention provided by a particular clasp

- Tooth factor

The size of the angle of the cervical convergence.	

How far is the clasp terminal is placed in angle of cervical convergence	

- flexibility of the clasp arm

- The length of the clasp arm.

- The diameter of the clasp arm.

- The cross-sectional form.

- The type of metal used.

# length of clasp arm

The length is measured from point of origin to the terminal end. The longer the clasp arm the more flexible it will be. (flexibility directly proportion to the cube of the length)

# Diameter of clasp arm:

The greater average diameter of the clasp arm the less flexibility it will be. (flexibility inversely proportion to the diameter of the clasp)

# Cross sectional form of the clasp arm:

Flexibility may exist in any form ( round, half round and other).

The cross sectional form affects flexibility.

A round clasp has greater flexibility than a half-round clasp with the same diameter. It also has the ability to flex in all plans, whereas a half round clasp normally flexes in only a single plane.

3 4

Because of the internal structure of wrought wire, it has greater ability to flex than cast alloy. To obtain equal retention, therefore, a greater depth of undercut is required for a wrought wire clasp than for a cast clasp.

2

3- stabilization or bracing

4-

reciprocal

5- encirclement

6-