قــســــــــــم الامــــــــــــــــــــــــــن الــــــــــــــــــــــــــــــــــــسيبرانــــــــــــــــــــــــــــــــــــي

Department of Cyber Security

MICROPROCESSORS

Lecture: (4)

microprocessor

Class: Second

Lecturer: M.Sc.Muntather AL-mussawee

Control & Status Signals:

ALE: Address latch enable

RD: Read control signal.

WR: Write control signal.

IO/M, S1 and SO: Status signals

Power Supply & Clock Frequency:

Vec: +5 V power supply

Vss: Ground reference

X1, X2: A crystal having frequency of 6 MHz is connected at these

two pins

CLK: Clock output

Externally Initiated and Interrupt Signals:

RESET IN: When the signal on this pin is low, the PC is set to 0,

the buses are tri-stated and the processor is reset.

RESET OUT: This signal indicates that the processor is being

reset. The signal can be used to reset other devices.

READY: When this signal is low, the processor waits for an

integral number of clock cycles until it goes high.

• HOLD: This signal indicates that a peripheral like DMA (direct

memory access) controller is requesting the use of address and data bus.

• HLDA: This signal acknowledges the HOLD request.

• INTR: Interrupt request is a general-purpose interrupt.

• INTA: This is used to acknowledge an interrupt.

• RST 7.5, RST 6.5, RST 5,5 - restart interrupt: These are vectored

interrupts and have highest priority than INTR interrupt.

• TRAP: This is a non-maskable interrupt and has the highest priority. •

Serial I/O Signals:

• SID: Serial input signal. Bit on this line is loaded to D7 bit of register

A using RIM instruction.

• SOD: Serial output signal. Output SOD is set or reset by using SIM

instruction.

INSTRUCTION SET AND EXECUTION IN 8086

Based on the design of the ALU and decoding unit, the microprocessor

manufacturer provides instruction set for every microprocessor. The

instruction set consists of both machine code and mnemonics.

An instruction is a binary pattern designed inside a microprocessor to

perform a specific function. The entire group of instructions that a

microprocessor supports is called instruction set. Microprocessor

instructions can be classified based on the parameters such functionality,

length and operand addressing.

Classification based on functionality:

1-Data transfer operations: This group of instructions copies data

from source to destination. The content of the source is not altered.

2- Arithmetic operations: Instructions of this group perform

operations like addition, subtraction, increment & decrement. One

of the data used in arithmetic operation is stored in accumulator

and the result is also stored in accumulator.

3- Logical operations: Logical operations include AND, OR, EXOR,

NOT. The operations like AND, OR and EXOR uses two

operands, one is stored in accumulator and other can be any

register or memory location. The result is stored in accumulator.

NOT operation requires single operand, which is stored in

accumulator.

4- Branching operations: Instructions in this group can be used to

transfer program sequence from one memory location to another

either conditionally or unconditionally.

5- Machine control operations: Instruction in this group control

execution of other instructions and control operations like interrupt, halt

etc.

Memory segmentation:

Segmentation is the process in which the main memory of the computer

is logically divided into different segment and each segment has it own

base address.